| Category | Technique / Method | Purpose / Insight | How it Applies to Compost System |
| --- | --- | --- | --- |
| Descriptive Math | Moving average / exponential smoothing | Reduce noise, reveal trends | Smooth temperature, humidity, gas readings to detect real changes vs sensor noise |
|  | Rate of change (ΔT, Δhumidity, Δgas) | Detect fast events | Identify microbial blooms (rapid temp rise) or anaerobic shifts (CH₄ rise) |
|  | Ratios & indices (CO₂/O₂, CH₄/O₂) | Condense multiple signals into one metric | Respiration index, anaerobic risk indicator |
| Statistical Analysis | PCA (Principal Component Analysis) | Find dominant dynamics | Combine temp, humidity, CO₂, CH₄ into “activity vs moisture” components |
|  | Cross-correlation | Find lag between signals | Determine if stirring or heating affects gas emission after N hours |
|  | Anomaly detection (Z-score, rolling thresholds, ML) | Detect sensor faults or abnormal compost states | Alerts when temp too high, humidity too low, or gases behave unexpectedly |
| Time-Series / Signal Processing | FFT / spectral analysis | Detect periodic patterns | Find diurnal effects, motor stirring cycles, or microbial oscillations |
|  | Autoregressive models (AR, ARIMA) | Forecast temperature, gas levels | Predict overheating or cooling, plan stirring/heating |
|  | Phase detection / clustering | Detect compost stages | Mesophilic → thermophilic → cooling transitions |
| Control Systems | PID control (Proportional–Integral–Derivative) | Maintain optimal temp | Heater control: maintain 55–65°C with feedback from temperature sensor |
|  | Bang–bang / On-Off control | Simpler temp or moisture management | Heater on/off or motor on/off when thresholds are crossed |
|  | Model Predictive Control (MPC) | Optimize multiple actuators over horizon | Predict temperature/gas and decide optimal heating + stirring schedule |
|  | State-based control / Finite State Machine | Handle compost phases & actuator logic | Motor runs during active phase, heater adjusts differently during cooling phase |
| Advanced Math / Signal Fusion | Kalman / Extended Kalman Filter | Fuse multiple sensor readings, predict true state | Smooth temp + humidity + gas, estimate internal pile temp not directly measured |
|  | Entropy / Approximate Entropy | Measure system “health” | Detect if microbial activity is dynamic (healthy) or flat (inactive/dead) |
|  | Time delay / transfer function modeling | Understand actuator effect lag | Model heater → temp response or motor → gas mixing response |